🏆 Tugas Minggu 3: The Ultimate High Availability Network

(VTP + EtherChannel + HSRP + OSPF Dual Homing + NAT)

🏢 Skenario: "Bank SecureConnect" - Core Layer Upgrade

Bank Anda tidak hanya butuh router yang tidak boleh mati, tapi juga jalur antar-switch yang cepat dan manajemen VLAN yang terpusat.

- Switch Redundancy: Link antar-switch HQ harus digabung (bundled) agar bandwidth naik 2x lipat dan redundan.

- VLAN Management: Admin malas membuat VLAN di setiap switch baru. Gunakan VTP agar VLAN tersinkronisasi otomatis.

🗺️ Topologi Fisik (Diperbarui untuk EtherChannel)

Perhatikan perubahan pada Poin 1:

1. Area HQ (Switching Core):

- Switch to Switch (EtherChannel Link): Hubungkan S-HQ-1 dan S-HQ-2 menggunakan DUA KABEL.

  - Kabel 1: Fa0/23 ke Fa0/23

  - Kabel 2: Fa0/24 ke Fa0/24

- Router to Switch:

  - R-HQ-1 (Gig0/0) -> kabel ke -> S-HQ-1 (Gig0/1).

  - R-HQ-2 (Gig0/0) -> kabel ke -> S-HQ-2 (Gig0/1).

2. Area WAN (Dual Homing):

- R-Branch punya dua kaki (Serial) ke R-HQ-1 dan R-HQ-2.

3. Area Internet:

- R-HQ-1 dan R-HQ-2 punya kabel masing-masing ke ISP.

🔢 Tugas 1: Perhitungan VLSM (Tetap Sama)

Network: 172.16.0.0/16

- VLAN 10 (Staff): 500 Host.

- VLAN 20 (Server): 50 Host.

- VLAN 50 (Branch): 20 Host.

- VLAN 99 (Mgmt): 10 Host.

- WAN Links (x4): Masing-masing 2 Host.

⚙️ Tugas 2: Konfigurasi Packet Tracer (Updated)

Kerjakan berurutan dari Layer 2 ke Layer 3 agar tidak pusing.

A. Layer 2: Switching Advanced (VTP & EtherChannel)

Ini adalah tambahan baru permintaan Anda.

- EtherChannel (LACP):

  - Pada S-HQ-1 dan S-HQ-2, konfigurasikan port Fa0/23 dan Fa0/24 menjadi satu Port-Channel 1.

  - Gunakan protokol LACP (mode active).

  - Setel interface port-channel 1 sebagai Trunk.

- VTP (VLAN Trunking Protocol):

  - Domain: BANK_HQ

  - Password: admin123

  - S-HQ-1: Set sebagai VTP Server.

  - S-HQ-2: Set sebagai VTP Client.

- Pembuatan VLAN:

  - Buat VLAN 10, 20, dan 99 HANYA di S-HQ-1 (Server).

  - Verifikasi: Cek di S-HQ-2 (Client), pastikan VLAN tersebut muncul otomatis (show vlan brief).

- Management IP:

  - Berikan IP Address (VLAN 99) pada SVI kedua switch.

B. Layer 3: Gateway Redundancy (HSRP)

Konsep "3 IP Address" (Fisik R1, Fisik R2, Virtual).

- Sub-interfaces: Buat sub-interface untuk VLAN 10, 20, 99 di R-HQ-1 dan R-HQ-2.

- Konfigurasi HSRP:

  - R-HQ-1: Active (Priority 150, Preempt).

  - R-HQ-2: Standby (Priority 100).

  - Virtual IP: Pastikan sama di kedua router.

- DHCP Server:

  - Buat DHCP Pool di router (bisa di R1 atau keduanya dengan split scope).

  - PENTING: Default Router di DHCP harus mengarah ke IP Virtual HSRP.

C. Routing: OSPF Dual Homing

- Advertise Network:

  - Masukkan semua network (LAN HQ, WAN, Branch) ke Area 0.

- Redundansi Branch:

  - Pastikan R-Branch memiliki dua neighbor OSPF.

  - Cek tabel routing branch (show ip route), pastikan ada jalur redundan.

D. Edge: Internet Access

- NAT Overload: Konfigurasi di kedua Router HQ agar PC VLAN 10 bisa ping ke 8.8.8.8 (Loopback ISP).

Network: 172.16.0.0/16

| HQ VLAN 10 (Staff) | 500 Host | Butuh subnetmask yang cukup besar (lebih dari /24). |
| --- | --- | --- |
| HQ VLAN 20 (Server) | 50 Host | Departemen Sumber Daya Manusia. |
| VLAN 50 (Branch) | 20 Host | Operasional Kantor Cabang. |
| HQ VLAN 99 (Mgmt) | 10 Host | Untuk manajemen perangkat Switch/Router (IT). |
| WAN Link R-HQ-1 ke R-Branch | 2 Host | Koneksi Point-to-Point OSPF. |
| WAN Link R-HQ-1 ke ISP | 2 Host | Koneksi Internet Utama. |
| WAN Link R-HQ-2 ke ISP | 2 Host | Koneksi Internet Cadangan. |
| WAN Link R-HQ-2 ke Branch | 2 Host | Koneksi Point-to-Point OSPF. |

Tabel VLSM

| Nama | Host | Prefix | Network | Range IP | Broadcast | Subnet |
| --- | --- | --- | --- | --- | --- | --- |
| HQ VLAN 10 (Staff) | 500 Host | /23 | 172.16.0.0 | 172.16.0.1 - 172.16.1.254. | 172.16.1.255 | 255.255.254.0 |
| HQ VLAN 20 (Server) | 50 Host | /26 | 172.16.2.0 | 172.16.2.1 - 172.16.2.62 | 172.16.2.63 | 255.255.255.192 |
| VLAN 50 (Branch) | 20 Host | /27 | 172.16.2.64 | 172.16.2.65 - 172.16.2.94 | 172.16.2.95 | 255.255.255.224 |
| HQ VLAN 99 (Mgmt) | 10 Host | /28 | 172.16.2.96 | 172.16.2.97 - 172.16.2.110 | 172.16.2.111 | 255.255.255.240 |
| WAN Link R-HQ-1 ke R-Branch | 2 Host | /30. | 172.16.2.112 | 172.16.2.113 - 172.16.2.114 | 172.16.2.115 | 255.255.255.252 |
| WAN Link R-HQ-1 ke ISP | 2 Host | /30 | 172.16.2.116 | 172.16.2.117 - 172.16.2.118 | 172.16.2.119 | 255.255.255.252 |
| WAN Link R-HQ-2 ke ISP | 2 Host | /30 | 172.16.2.120 | 172.16.2.121 - 172.16.2.122 | 172.16.2.123 | 255.255.255.252. |
| WAN Link R-HQ-2 ke Branch | 2 Host | /30 | 172.16.2.124 | 172.16.2.125 - 172.16.2.126 | 172.16.2.127. | 255.255.255.252 |

S-HQ2#sh run

Building configuration...

Current configuration : 1815 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S-HQ2

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name BANK_HQ

!

username admin privilege 15 password 0 admin123

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/1

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

switchport trunk native vlan 99

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

!

interface FastEthernet0/24

switchport trunk native vlan 99

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.2.100 255.255.255.240

!

banner motd ^C

Switch HQ 2, Unauthorized user can't login ^C

!

!

!

line con 0

password admin123

login

!

line vty 0 4

login local

transport input ssh

line vty 5 15

login

!

!

!

!

End

S-HQ1#sh run

Building configuration...

Current configuration : 1814 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S-HQ1

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name BANK_HQ

!

username admin privilege 15 password 0 admin123

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

switchport trunk native vlan 99

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

!

interface FastEthernet0/24

switchport trunk native vlan 99

switchport mode trunk

channel-protocol lacp

channel-group 1 mode active

!

interface GigabitEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.2.99 255.255.255.240

!

banner motd ^C

Switch HQ 1, Unauthorized user can't login ^C

!

!

!

line con 0

password admin123

login

!

line vty 0 4

login local

transport input ssh

line vty 5 15

login

!

!

!

!

end

-Branch#sh run

Building configuration...

Current configuration : 1434 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S-Branch

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name Branch.local

!

username admin privilege 15 password 0 admin123

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport mode trunk

!

interface FastEthernet0/2

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C

Switch Branch, Unauthorized user can't login ^C

!

!

!

line con 0

password admin123

login

!

line vty 0 4

login local

transport input ssh

line vty 5 15

login

!

!

!

!

End

ISP#sh run

Building configuration...

Current configuration : 1451 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname ISP

!

!

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin privilege 15 password 0 admin123

!

!

license udi pid CISCO2911/K9 sn FTX1524LPSB-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name ISP.local

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback0

ip address 8.8.8.8 255.255.255.255

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 172.16.2.122 255.255.255.252

!

interface Serial0/0/1

ip address 172.16.2.118 255.255.255.252

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 3.3.3.3

log-adjacency-changes

network 172.16.2.116 0.0.0.3 area 0

network 172.16.2.120 0.0.0.3 area 0

network 8.8.8.8 0.0.0.0 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd ^C

Router ISP, Unauthorized user can't login ^C

!

!

!

!

line con 0

password admin123

login

!

line aux 0

!

line vty 0 4

login local

transport input ssh

!

!

!

End

R-HQ1#sh run

Building configuration...

Current configuration : 2498 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R-HQ1

!

!

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

ip dhcp excluded-address 172.16.0.1 172.16.0.3

ip dhcp excluded-address 172.16.0.101 172.16.1.254

ip dhcp excluded-address 172.16.2.1 172.16.2.3

ip dhcp excluded-address 172.16.2.31 172.16.2.62

!

ip dhcp pool Staff

network 172.16.0.0 255.255.254.0

default-router 172.16.0.2

dns-server 8.8.8.8

ip dhcp pool Server

network 172.16.2.0 255.255.255.192

default-router 172.16.2.2

dns-server 8.8.8.8

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin privilege 15 password 0 admin123

!

!

license udi pid CISCO2911/K9 sn FTX1524A0VK-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name BANK_HQ

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 172.16.0.1 255.255.254.0

standby 1 ip 172.16.0.2

standby 1 priority 150

standby 1 preempt

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 172.16.2.1 255.255.255.192

standby 1 ip 172.16.2.2

standby 1 preempt

!

interface GigabitEthernet0/0.99

encapsulation dot1Q 99 native

ip address 172.16.2.97 255.255.255.240

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 172.16.2.113 255.255.255.252

!

interface Serial0/0/1

ip address 172.16.2.117 255.255.255.252

ip nat outside

clock rate 2000000

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 172.16.0.0 0.0.1.255 area 0

network 172.16.2.0 0.0.0.63 area 0

network 172.16.2.112 0.0.0.3 area 0

network 172.16.2.116 0.0.0.3 area 0

!

ip nat inside source list 10 interface Serial0/0/1 overload

ip classless

!

ip flow-export version 9

!

!

access-list 10 permit 172.16.0.0 0.0.1.255

access-list 10 permit 172.16.2.0 0.0.0.63

access-list 10 permit 172.16.2.64 0.0.0.31

access-list 10 permit 172.16.2.112 0.0.0.3

!

banner motd ^C

Router HQ 1, Unauthorized user can't login ^C

!

!

!

!

line con 0

password admin123

login

!

line aux 0

!

line vty 0 4

login local

transport input ssh

!

!

!

End

R-HQ2#sh run

Building configuration...

Current configuration : 2471 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R-HQ2

!

!

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

ip dhcp excluded-address 172.16.0.1 172.16.0.3

ip dhcp excluded-address 172.16.0.4 172.16.0.100

ip dhcp excluded-address 172.16.2.1 172.16.2.3

ip dhcp excluded-address 172.16.2.4 172.16.2.30

!

ip dhcp pool Staff

network 172.16.0.0 255.255.254.0

default-router 172.16.0.2

dns-server 8.8.8.8

ip dhcp pool Server

network 172.16.2.0 255.255.255.192

default-router 172.16.2.2

dns-server 8.8.8.8

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin privilege 15 password 0 admin123

!

!

license udi pid CISCO2911/K9 sn FTX152428AO-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name BANK_HQ

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 172.16.0.3 255.255.254.0

standby 1 ip 172.16.0.2

standby 1 preempt

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 172.16.2.3 255.255.255.192

standby 1 ip 172.16.2.2

standby 1 preempt

!

interface GigabitEthernet0/0.99

encapsulation dot1Q 99 native

ip address 172.16.2.98 255.255.255.240

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 172.16.2.125 255.255.255.252

!

interface Serial0/0/1

ip address 172.16.2.121 255.255.255.252

ip nat outside

clock rate 2000000

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

network 172.16.0.0 0.0.1.255 area 0

network 172.16.2.0 0.0.0.63 area 0

network 172.16.2.124 0.0.0.3 area 0

network 172.16.2.120 0.0.0.3 area 0

!

ip nat inside source list 10 interface Serial0/0/1 overload

ip classless

!

ip flow-export version 9

!

!

access-list 10 permit 172.16.0.0 0.0.1.255

access-list 10 permit 172.16.2.0 0.0.0.63

access-list 10 permit 172.16.2.64 0.0.0.31

access-list 10 permit 172.16.2.124 0.0.0.3

!

banner motd ^C

Router HQ 2, Unauthorized user can't login ^C

!

!

!

!

line con 0

password admin123

login

!

line aux 0

!

line vty 0 4

login local

transport input ssh

!

!

!

End

R-Branch#sh run

Building configuration...

Current configuration : 1399 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R-Branch

!

!

!

enable secret 5 $1$mERr$AFX/pZT1Lh7NP3Dp3P/qq/

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin privilege 15 password 0 admin123

!

!

license udi pid CISCO2911/K9 sn FTX152429AY-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name Branch.local

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.50

encapsulation dot1Q 50

ip address 172.16.2.65 255.255.255.224

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 172.16.2.114 255.255.255.252

clock rate 2000000

!

interface Serial0/0/1

ip address 172.16.2.126 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 4.4.4.4

log-adjacency-changes

network 172.16.2.64 0.0.0.31 area 0

network 172.16.2.124 0.0.0.3 area 0

network 172.16.2.112 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd ^C

Router Branch, Unauthorized user can't login ^C

!

!

!

!

line con 0

password admin123

login

!

line aux 0

!

line vty 0 4

login local

transport input ssh

!

!

!

end